Digital Image Processing

### Ingenious Governance of Battle Tanks

### Using

### Microcontroller Based GPS Tracking

BY

Author 1                                                     Author 2

K.NAVYA                                                G.PRASANTHI

Konde.navya@gmail.com                       Prasanthi780@gmail.com

Mobile no: 9912884294                            Mobile no:9703418603

B TECH IIIrd YEAR

ELECTRONICS AND COMMUNICATION ENGINEERING

KONERU LAKSHMAIAH COLLEGE OF ENGINEERING

## ABSTRACT:

## The main objective is to automate the vehicles so that they relieve us of our headache in sneaking our way through heavy traffic. As a primary step, this automation is implemented in a battle tank where driving the vehicle is relatively simple. Besides, the battle tank does not have a steering.  All that the battle tank uses for its driving are the two tracking sticks.

In this project we the GPS receiver 9312B for navigational management features to guide the tank through the path that it is supposed to tread. The output of the receiver is fed to the microcontroller which is programmed in such a way so as to compare whether the range is zero. When the range is zero it deciphers the turn it has to undertake. The required turn is accomplished with the microcontroller generating the appropriate byte to control the stepper motor. The microcontroller decides the left or right turn using a special KRASS algorithm. This versatile algorithm exploits the NMEA details of the GPS receiver and makes those details compatible with the microcontroller unit. The microcontroller promptly commands the stepper motor to rotate in the desired direction. The direction the battle tank twirls is also displayed in a LCD for the commander’s convenience. This paper when implemented could assuage the miseries in driving a tank.

## INTRODUCTION:

An innovative channel for an age-old concept is what this project is all about. The receiver at the top of the tank helps to navigate the vehicle taking into consideration the proceeding and the following tanks. A GPS receiver is used to plot the precise location of tank. The destination of the tank is sketched and the direction the tank should move is passed as control signal by the microcontroller (8952).The stepper motor is controlled with the aid of the following data from the GPS receiver

- Range.

- Trackling angle.

- steering angle

- cross track error

- group speed

The way points where the tank should change path are fed to the GPS receiver. When that particular way point is encountered the microcontroller comprehends that the tank should turn and activates the particular lever. The LAT/LON position of the places where the tank should stop is given as the way-point. When the place is reached the stepper motor is given control signal to activate the lever for the certain amount of time summoned by a delay routine.

GLOBAL POSITIONING SYSTEM:

GPS refers to the group of 24 satellites rotating around the earth at an altitude of 20,200 km. above the mean sea level. This helped to locate any receiver at any point on the earth. The GPS receives the communication from the satellites and know the way we wish it to be.

The GPS receiver we use is GPS 9321B, a satellite based global positioning and navigational system that works on GPS satellite constellation. All the satellites of the GPS transmit on 1575.42 MHz (L1) with CV/N coded 1025 KHz data and the receiver works on the signal. This GPS receiver effectively provides following details:

User position information, Local time and UTC.

Navigational management features.

IGRS, Lat/Lon or UTM modes with previous position set with default mode.

Receiver system designed to provide acquisition on starter with fast      updating. Flash RAM to save power.

IGRS provides current heading, time, date, ground speed, etc.

Built in signal processing uses correlation techniques to get navigational information.

Computed information is available in RS232 port.

SATELLITE   CONFIGURATION:

GPS NOMINAL CONSTELLATION

24 satellites in 6 orbital planes, 4 satellites in each plane

20,200 km altitudes, 55 degree inclination

This project exploits the data available on this RS232 port. In GPS system the position of installation in Lat/Lon, geodal height etc., will be provided by satellite constellation as NMEA 0183 standard data protocol. This information shall be available on the RS232 port of the GPS receiver. It communicates @ 4800 bps with one stop bit and with parity none. For the GPS receiver to transmit the NMEA strings through the RS232 port the NMEA mode should be switched on.

The determination of position in a GPS may be described as the process of triangulation using the measured range between the user and four or more satellites. The ranges are inferred from the time of propagation of the satellite signals. Four satellites are requested to determine the three coordinates of position and time. The time is involved in the correction to the receiver clock and is ultimately eliminated from the measurement of position. High precision through the use of atomic clocks carried on – board the satellites. The design of the GPS constellation had the fundamental requirement that at least four satellites must be visible at all times from any point on earth. The orbital configuration approved in 1973 has 24 satellites, consisting of 8 satellites plus one spare in each of three equally spaced orbital planes. The orbital radius was 26,562 km, corresponding to a period of revolution of 12 sidereal hours, with repeating ground traces. A common orbital inclination of 63 was selected to maximize the on-orbit payload mass with launches from the western test range. This

configuration ensured between 6 and 11 satellites in view at any time.

MEASUREMENTS IN GPS:

PSEUDO RANGE:

The fundamental measurement in the GPS is pseudo range. The user equipment receives the PRN code from a satellite and having identified the satellite, generates a replica code. The phase by which the replica code must be shifted in the receiver to maintain maximum relationship with the satellite code, multiplied by the speed of light, is approximately equal to the satellite range. It is called the pseudo range because the measurement must be corrected by a variety of factors to obtain the true range. The Kalman filter has led to improved performance in measuring the true range which is obtained by eliminating the errors.

CARRIER PHASE:

Carrier phase is used to perform measurements with a precision that greatly exceeds those based on pseudo range. However, a carrier phase measurement must resolve an internal cycle ambiguity whereas the pseudo range is unmistakable.

SIGNAL STRUCTURE:

The satellite position at any time is computed in the user’s receiver from the Navigation message that is contained in a 50bps data stream .the orbit is represented for each one hour period by a set of keplerian orbital elements with a Harmonic coefficient arising perturbations, and is updated every four hours.

The data stream is modulated by each of two code division multiple access, spread spectrum pseudorandom noise (PRN) codes; the coarse / acquisition C/A code(Sometimes called the clear access code) and the precision p code. The p code can be encrypted to produce the secure code called the y code, this feature is known anti-spoof (AS) mode. This is intended to defeat deception jamming by adversaries. The C/A code is used for satellite acquisition and for position determination by civil receivers. The P(Y) Code is used by military and other authorized receivers.

MICROCONTROLLER 89C52:

The 8952 microcontroller is the 8951’s “big brother”. It is a slightly more powerful microcontroller, sporting a number of additional features:

256 bytes of internal RAM (compared to 128 in the standard 8951).

A third 16 bit timer, capable of a number of new operation modes and 16 bit reloads.

Additional SFRs to support the functionality offered by third timer. The main feature is its integrated UART, otherwise known as serial port. The fact is that it is very easy to read and write values to the serial port.	The microcontroller holds the program to decide the direction the tank has to be navigated. The Lat/Lon position to turn from the initial point, the right or left turn to be moved, the angle of rotation to change etc., are implied in the program itself. The conditions are checked and the appropriate decision is taken by the microcontroller program. The resulting information is given to the stepper motor to turn to the exact angle. The microcontroller needs some interfacing device to drive the stepper motor which is needed to turn the angle of navigation of the tank.

The C/A code is gold code of register size 10, which has the sequence length of 1023 chips and the chipping rate of 1.023 MHZ and repeats its self every one Milliseconds. (The term “chip” is used instead of “bit” to indicate that the PRN code contains no information) the p code is a long code is a length 2.3547*1014 chips with a chipping rate of 10 times the C/A code, or 10.23 MHZ. at this rate the p code has the period of 38.058 weeks, but it is truncated on a weekly basis so that 38 segments are available for constellation. Each satellite uses a different member of C/A gold code family and a different one-week segment of the p code sequence.

FLOW CHART FOR THE CONTROL OF AUTOMATED                                             BATTLE TANK USING GPS RECEIVER:

ANTENNA

GPS RECEIVER

DECIDE THE DIRECTION MICROCONTROLLER UNIT

STEPPER MOTOR

KRASS ALGORITHM -FLOW CHART:

START

SET BAUD RATE       FOR 89C52

MOVE THE RECEIVED ASCII DATA FROM SBUF TO ACC

COMPARE WHETHER RANGE IS ZERO OR NOT

COMPARE WPT RIGHT OR LEFT

## RIGHT

## TURN

## LEFT TURN

# FUTURE ENHANCEMENTS:

Our paper on EMBEDDED SYSTEM with Automation of Battle Tank Driving Using GPS Receiver can also be enhanced in a superior way in future. The following are some of the steps to be implemented for a new era of unmanned tank in upcoming future

- Speed control of the tank

- Control of gear mechanism

- Automatic tracking.

# CONCLUSION:

This paper when implemented as a project, the battle tanks could act in case of emergencies. Supposing the driver is injured during the course of their journey, we cannot expect him to urge to a nearby hospital and a substitute driver to take in charge of the vehicle in a battle field. In such cases the data available on the GPS receiver could be exploited and used to automatically navigate the battle tank.

REFERENCES:

[1] D. M. Gavrila, “GPS TRACKING USING MICROCONTROLLERS”, Proc. of European Conference on Computer

Vision, pp. 37-49, Dublin, Ireland, 2000

[2] I. Ulrich and I. Nourbakhsh, “MICROCONTROLLERBASED BATTLE TANK GOVERNANCE”, Proc. of the AAAI

National Conference on Artificial Intelligence, Austin, TX,